Northwest Power & Conservation Council

BIENNIAL REVIEW OF THE COST OF WIND POWER

July 13, 2006

The Northwest Power and Conservation Council, in its 5th Power Plan estimated the levelized cost of new utility-scale windpower to range from $42 to $53/MWh.  The assumptions upon which these costs are based, developed with the assistance of an advisory group comprised of industry and utility representatives, are shown in the following table.  The assumptions date from 2002, a time of moderate wind development activity and were thought to be representative of equilibrium market conditions, suitable for the long-term nature of the Council’s Plan.  Conditions through late 2004, when the 5th Plan was adopted, did not appear to deviate significantly from these assumptions.

|  | Year 2000 (2006) dollars |
| --- | --- |
| Project Size (MW) | 100 |
| Capital ($/kW) | $1010 ($1160) |
| Fixed O&M ($/kW/yr) | $20 ($23) |
| Variable O&M ($/MWh) | $1 ($1.15) |
| Capacity Factor (%) | 28 - 30% |
| Shaping & Integration ($/MWh) | $4.55 - $9.75 ($5.23 - $11.20 |
| Wheeling ($/kW/yr) | $20 ($23) |
| Transmission Losses | 1.9% |
| Project life | 20 years |
| Learning effects (on real cost) | -2.2 %/yr |

The cost of new wind projects has risen substantially in real terms over the past two years.  Bids for shaped and delivered energy from projects entering service in 2006 or 2007 range from about $45 to over $100/MWh.  The principal element leading to the increase in delivered energy cost is an average real increase in project construction cost of about 40 to 50% over the assumption of the 5th Plan.  Offsetting this cost increase has been an improvement in energy capture and conversion efficiency of about 7%.

Factors contributing to recent cost increases

Factors at play include the following:

Weakening dollar:  Major components of many of the wind turbine generators used in U.S. wind farms are manufactured overseas.  A weakening U.S. dollar has increased the cost of these machines.  For example, the value of the Euro against the dollar has increased from $0.98 in July 2002 to $1.21 in March 2006.

Increased commodity and energy costs: Commodities used in the manufacture and installation of wind turbines and ancillary equipment, including cement, copper, steel and resin (for blades) have increased in cost in recent years.  Drivers have included general economic recovery, disaster recovery and increased demand from developing Asian economies.  NYMEX copper increased from $0.72/lb in July 2002 to $2.32/lb in March 2006.  Rebar has increased about 45% over the same period.  Structural concrete is forecast to increase to about $580/cy in 2006, up 50% from 2002.  Likewise, the cost of energy needed to fabricate, transport and erect wind turbine generators and related components has also increased.  The average U.S. retail price of No. 2 diesel has increased from $0.85/gallon in July 2002 to $2.07/gallon in March 2006.  Some believe that commodity prices are leveling off as new supply is developed or demand weakening.  Copper futures, for example are declining.

Market demand for windpower:  High natural gas costs, pending expiration of the federal production tax credit at the end of 2007, adoption of state renewable portfolio standards and increasing utility recognition of the risk of future CO2 control costs have increased demand for new windpower.  Increased demand has created shortages of turbines, specialized transportation and erection equipment, and experienced construction workers and operations and maintenance personnel.  The buyer’s market may have encouraged increased profit taking where possible among players in an industry that has experienced many lean years, though margins are said to remain thin.  Turbines are generally not available from established vendors through 2007, last year of the current PTC extension, but are available for 2008 delivery.

Financing:  Changes in the structure and terms of project financing have occurred, motivated by a maturing industry, the federal production tax credit and accelerated depreciation rates.  Financing trends include (1) increasing investor-owned utility ownership of projects; (2) lower debt fractions including unlevered (pure equity) financing for non-utility projects; (3) lower equity return among investors in non-utility projects and (4) emergence of complex financial structures for the purpose of transferring PTC and accelerated depreciation tax benefits to third party investors able to use these benefits.

Performance improvements: Increases in swept area and hub heights, improved reliability and improved project and turbine siting have lead to improvement in turbine performance.  Though not a completely reliable indicator of improved performance, the energy capture and conversion efficiency of recent and proposed wind projects appear to be about 7 percent above the 5th Plan base assumption.  In absolute terms, the typical capacity factor of a project located in eastern Washington or Oregon has improved from 30% to about 32%.  The current generation of terrestrial utility-scale machines is approaching transportation and erection size and weight limits for prefabricated nacelles, blades and tower sections.  This may reduce the historical rate of performance improvement.

Project size:  Increasing project size has produced economics of procurement, construction and operation.  Average project size in the Northwest increased from about 20 MW for projects constructed from 1997 to 2000 to over 175 MW for projects currently under construction.  Total project size continues to increase with the latest announced Northwest project being 750 MW.  This and other very large projects are likely to be developed in phases, however.

Other cost elements:  Non-capital cost elements including operating and maintenance, wheeling and shaping costs appear to have remained fairly constant.   A constrained supply of ancillary services for project integration may lead to future increases in the cost of integration.

Analysis of current capital costs

The plot of wind project capital costs shown below was prepared using a combination of announced capital costs of utility-scale projects completed or under construction from 1997 through 2005, “back-calculated” capital costs from recent bids and information regarding current turbine and full project costs. The back calculations were performed under the assumption that the primary factors leading to recent cost increases have been capital costs, using financial assumptions generally representing current practice.   Capacity factors were assumed to average 32%.

The capital costs shown in the figure are all-in project costs excluding financing, debt reserve and other transaction costs.  Two data series are shown.  The pale “Full series” series includes all sources available for this estimate.  The darker “higher confidence” series is limited to data points thought to be more reliable or more representative of current and near-term conditions (for example, costs based on the upper 50% of bid results were omitted under the assumption that these projects are unlikely to be selected for development.  Likewise older and small projects were omitted).  The trend line is derived from the higher confidence series.

The resulting plot shows the decline in wind project capital costs through 2003.   The subsequent increase in capital costs under the influence of the factors described above is clearly evident.  The 2006 costs have an unweighted average of $1485/kW and a moderaely wide range of less than $1300 to over $1700/kW.  Larger project size, larger (multi-project) turbine orders and transportation and construction equipment reservations well in advance of need are factors leading to lower cost projects, whereas smaller projects, per-project turbine orders and less-efficient construction management will lead to higher costs.

Proposed assumptions

The assumptions shown in the following table are proposed for purposes of characterizing typical near-term utility-scale projects.  Average project size is assumed to have increased to 150 MW, based on recent Northwest projects.  Overnight capital costs, including project development, owner’s and typical transmission interconnection costs, range from about $1100 to $1900/kW and are assumed to average $1500/kW.  Fixed and variable O&M costs remain at the levels adopted in the 5th Plan.  The typical capacity factor of a project located in the inland Northwest, based on energy to the point of interconnection is assumed to have increased to 32%.  Wheeling and integration costs, wheeling losses and project life are unchanged from 5th Plan estimates.

|  | Year 2006 dollars |
| --- | --- |
| Project Size (MW) | 150 |
| Capital ($/kW) | $1300 - 1700 |
| Fixed O&M ($/kW/yr) | $20 |
| Variable O&M ($/MWh) | $1.00 |
| Capacity Factor (%) | 32% (Eastern WA or OR) |
| Shaping & Integration ($/MWh) | $4.50 - $10.00 ($4.50 in near-term) |
| Wheeling ($/kW/yr) | $20 |
| Transmission Losses | 1.9% |
| Project life | 20 years |
| Learning effects (on real cost) | None for near-term |

Changes in financing and interest rates suggest that assumptions regarding certain financing assumptions should be changed as well.  11% appears to be a more representative IOU return than the 12% used previously.  The increasing availability of capital seeking PTC and accelerated depreciation tax benefits has lowered the post-tax equity returns for non-utility investors to about 8.5%.  Non-utility debt levels have declined to 40% (or less) because of the amount of equity needed to fully secure the federal tax benefits of project ownership.

These assumptions yield a levelized energy costs for near-term projects of $72 to $98/MWh (fully shaped and delivered to the load-serving utility).  Larger projects at better sites with more experienced developers will tend toward the lower portion of this range.  The cost of projects actually developed will likely be in the lower portion of this range because the more favorable proposals will be the ones selected for development in the near-term.

The proposed assumptions are currently intended for use in near-term analyses and do not constitute revisions to the 5th Plan.  They will be considered in the biennial review of the plan.

2006 dollars, 2005 service, shaped and delivered, inclusive of the federal production tax credit for private financing.

Costs are expressed in year 2000 dollars in the 5th Power Plan.  Values in parenthesis are the year 2006 dollar equivalents to facilitate comparison to current market conditions.

Excluding property tax and insurance.

Eastern Washington, Oregon and Idaho sites.  MT - 36%.